# Tutorial 1

Screen 1

This first tutorial demonstrates the operation of a direct-mapped cache and provides an

introduction to the HASE Dinero simulator.  Before commencing this tutorial, ensure

that you have the Unified Cache Contents window open (see the user guide if you are

unsure how to do this).  During the course of the simulation, the Status screen (in the

top right-hand corner of the window) will indicate the status of current simulation

activity from the point of view of the cache.  When you are ready to continue, click on

the single-step icon in the Animation window (a yellow icon showing a page with a 1

over the top left-hand corner).

Screen 2

The first step in this simulation shows the CPU issuing a data read command to the

cache.  The Data Read packet you just observed passing from the CPU to the Cache

Control Unit contains an address value that will be split up into its component parts

by the Cache Control Unit during the next simulation time-step.  Notice that the

CACHE Hit Type section of the status window has changed to WAIT.  This will be

updated to HIT or MISS when the Cache Control Unit has determined whether a

cache hit has occurred.

Screen 3

The Status window has now updated to indicate that the current CPU address is

0.  An address of 0 obviously produces Tag, Index and Block Offset components

that are all 0.  For an explanation of how the Tag, Index and Block Offset

components are calculated from the address, see the HASE Dinero user-guide.

Notice also that the Status screen now shows that program line 1 is being processed

and the CPU access type is DATA_READ.  The Line Read Packet just sent by the

Cache Control Unit to the Cache Memory Array contains the current Index (0).

Screen 4

The Cache Memory Array responds to the Line Read command from the Cache Control

Unit by sending the contents of associative set 0.  As the cache in this example is direct-

mapped, this requires sending just one cache line (line 0).  However, if the cache was, for

example, 4-way set-associative then the Cache Memory Array would send the contents of

all 4 lines in the associative set to the Cache Control Unit at this stage.

Screen 5

The Cache Control Unit checks the content of line 0 and observes that the valid bit (the

first value stored in line 0 of the cache) is set to 0 (invalid).  As a consequence, the

Cache Control Unit sends a Block Read command to main memory, requesting a copy

of the block containing the CPU Word at address 0.  As the Cache Control Unit has now

determined that a cache hit has definitely not occurred, the Cache Hit section of the

Status screen has been set to NO.

Screen 6

Main memory responds to the Block Read request by sending the requested block to the

Cache Control Unit.  Blocks are the atomic unit of cache to main memory transfers and

usually contain more than 1 CPU word.  However, in this example the block size has

been set to 1 byte, and so each cache line contains only 1 CPU Word (HASE Dinero

uses a CPU word size of 1 byte).

Screen 7

The Cache Control Unit has sent the requested word to the CPU.  In addition, you will

notice that Line 0 of the Unified Cache Contents has been modified.  The 1st element, the

VALID BIT, has been set to 1 to indicate that valid data is stored in the cache line.  The

2nd element, the MODIFIED BIT, has been left at 0, because the block stored in line 0 is

consistent with main memory.  The 3rd element, the TIMESTAMP, has been set to the

current simulation time of 6.  The 4th element, the TAG, has been set to the tag value (0)

of the block stored in line 0.  The 5th element is the sequence number of the first

byte in the block stored in cache line 0.

Screen 8

The CPU, having just received the data in response to its previous memory reference,

now sends a new request to the cache: an INSTRUCTION FETCH.  The instruction

fetch request is exactly the same as a data read request, in that a hexadecimal address

is supplied for which the CPU requires the relevant data.

Screen 9

Notice that the Status window has been updated to indicate the current address and its

breakdown.  The following four simulation steps are exactly the same as for the previous

CPU data read request, and are provided to give you a chance to familiarise

yourself with the Cache Miss procedure.  However note that the block from main

memory is loaded to cache line 1 this time.  If you are unclear on how the Tag, Index

and Block Offset components are calculated from the CPU address, then have a look in

the HASE Dinero user guide, which provides an illustrated example.

Screen 10

The CPU has just sent another Data Read request to the cache.  However, this time the

request will demonstrate what happens on a cache hit.  Notice that when the Cache

Control Unit writes data to the Cache Memory Array, the WRITE LINE SELECT

field in the Status window indicates which cache line has been selected for writing.

Screen 11

The Cache Control Unit checks the line just read from the Cache Memory Array and

finds that the valid bit is set to 1.  It therefore checks the tag field and finds that it is set

to 0, which corresponds to the tag field of the current address request from the CPU.

The Cache Control Unit now knows for certain that a cache hit has occurred (notice

that the Status window Cache Hit box has been updated to YES).  The Cache Control

Unit therefore sends the requested data to the CPU and updates the timestamp of cache

line 0, setting it to 16 (the current simulation time).

Screen 12

The CPU has just sent a Data Read request to the Cache Memory Array with an address

of 8.  As the cache in this example has just 8 cache lines, each containing only 1 byte,

this request will demonstrate how multiple addresses map to the same cache line.

Screen 13

Having loaded the required block from main memory, the Cache Control Unit has just

sent the required data to the CPU and updated the Cache Memory Array.  As the index

component of address 8 is the same as for address 0, both addresses are mapped to the

same cache line.  However, address 0 has a tag of 0 whereas address 8 has a tag of 1.

Consequently, line 0 of the Unified Cache Contents now has had the tag field updated

from 0 to 1.  In addition, the byte sequence number (at the end of cache line 0) has been

updated to 3 from 1 to indicate that a new byte has been stored.

Screen 14

This is the end of tutorial 1.  Hopefully you will have gained an understanding of how

a direct-mapped cache works and become more familiar with using HASE Dinero.  If

you cannot remember some of the aspects of cache operation shown in this tutorial, it is

recommended that you go through it again before proceeding to later tutorials, which

all assume knowledge of details covered in this tutorial.

Tutorial 2

Screen 1

Tutorial 2 demonstrates the difference between a direct-mapped cache and a set-

associative cache.  In this example, the cache is 2-way set associative.  This means

that for every CPU address the cache can choose between 2 cache lines in which

to store the address block.  You will need to open the Unified Cache Contents

window for this tutorial.  The tutorial assumes that you are familiar with the concepts

introduced in tutorial 1; the same address references are issued by the CPU in this

tutorial as tutorial 1 so that you can view the benefit of a 2-way associative cache over

a direct-mapped cache.

Screen 2

The CPU begins by issuing a data read command.  The address of the data read

command is 0 and so the index component is also 0.  However, as the cache is 2-way

set associative, the block that is loaded from main memory to service the cache miss

can be stored in either of the first two lines of the cache.  Hence the first two lines of

the cache can be referred to as the 0th Associative Set, the next two lines as the 1st

Associative Set and so on.

Screen 3

Note that, unlike the direct-mapped cache, the Cache Memory Array sends two cache

lines to the Cache Control Unit for each read request.  The two lines correspond to

the associative set selected by the Current Index of the CPU Address (sent to the

Cache Memory Array by the Cache Control Unit with the line read command).  The

remainder of this section of the tutorial shows the required block being loaded from

main memory to the Cache Memory Array and the requested data being sent to the

CPU.

Screen 4

The CPU now issues an instruction fetch command with an address of 1.  The index

component of this address is also 1, and so the block that is loaded from main memory

to service the cache miss can be stored in either line 2 or line 3 of the cache – the 1st

Associative Set.  The remainder of this section of the tutorial shows the required block

being loaded from main memory to the Cache Memory Array and the requested data

being sent to the CPU.

Screen 5

The CPU now issues a data read command with an address of 0.  As the block

corresponding to this address has already been loaded to the cache (in program

line 1), it will cause a cache hit.  Note that the Cache Memory Array sends 2

cache lines to the Cache Control Unit.  Therefore, the Cache Control Unit checks

both of the lines to see if either has both the valid bit set and the tag component

set to 0.  In this case, the first cache line of the associative set fulfils the criteria

and so a cache hit occurs.

Screen 6

The final demonstration in this tutorial shows the cache servicing a CPU address

of 8.  You will remember that in the previous tutorial, this caused a block already

stored in line 0 of the cache to be overwritten, because the cache was direct-mapped.

However, as the cache is 2-way set associative in this example, the Cache Control Unit

chooses to store the block for address 8 in the second line of the 0th Associative Set, as

its valid bit is not set.  Note that the Tag component of address 8 is 2 this time (instead

of 1 as in tutorial 1).  This is because the cache only needs an Index range of 4 (due to

the 2-way associativity), instead of 8 as for an equivalent direct-mapped cache.

Screen 7

This is the end of tutorial 2.  Hopefully, you will have gained an understanding of

the difference between a direct-mapped cache and a set-associative cache.  Note

that if the cache has a higher associativity (for example: 4) then there would be an

even larger range of cache lines that the Cache Control Unit could choose to load a

block to.  This will result in less overwriting of useful data in the cache.  The next

tutorial demonstrates the difference between a set-associative cache and a fully-

associative cache.

Tutorial 3

Screen 1

This tutorial demonstrates how CPU addresses are mapped to cache lines in a

fully-associative cache.  The same sequence of memory addresses as in the

previous 2 tutorials is used, so that a comparison between direct-mapped, set-

associative and fully-associative caches can be made.  You will need to open

the Unified Cache Contents window before starting the tutorial.

Screen 2

Although the sequence of CPU addresses is the same as the last tutorial, note that

because the cache is fully-associative there is only one available Index field: 0.

In addition, because the cache block size is set to 1, the Tag component of all

CPU addresses is exactly the same as the actual address.  During the course of

the demonstration, you should observe that all of the blocks loaded from main-

memory are stored sequentially in the Cache Memory Array.  This is because

all of the lines in the cache are eligible to be loaded with the block from ANY

CPU address.

Screen 3

This is the end of tutorial 3.  You should now understand the difference between

direct-mapped, set-associative and fully-associative caches.  You may wonder

why any cache design would not be fully-associative.  The answer is that for a

real cache implementation in hardware, the larger the associative set size is, the

more complex (and slower) the cache control unit becomes.  Hence there is a trade-off

to be made.  Note also that later tutorials investigate cache operation in more detail and

it is assumed that you understand the concepts of the first three tutorials.

Tutorial 4

Screen 1

This tutorial demonstrates the benefit to be gained by increasing the cache block

size, in terms of the cache hit ratio.  An 8 byte, direct-mapped cache with a block

size of 2 is used in this example (the block size has been set to 1 in the previous

3 tutorials).  You will need to open the Unified Cache Contents window before

proceeding.

Screen 2

During the course of this tutorial, four data read references are provided to the

cache by the CPU.  Each sequential data read issued by the CPU has an address

value 1 higher than the previous request, starting from address 0.  You will

notice that there are 2 cache hits and 2 cache misses.  This reason for this will

be investigated in more detail later in the tutorial.

Screen 3

The data read request from the CPU with an address of 0 has caused a cache miss.

As a consequence, the block containing byte address 0 has been loaded to the

Cache Memory Array.  However note that as the block size is set to 2, the byte

corresponding to byte address 1 has also been loaded to the cache (as the second

byte in the loaded block) and is now stored in cache line 0 (note that bytes are

represented by main memory access sequence numbers, NOT by the actual address

of the relevant byte).  The tutorial text will next update when the next CPU data read

request has been resolved.

Screen 4

You will have noticed that the data read request with an address of 1 has caused a

cache hit in the block loaded previously for memory address 0.  If the block size

had been set to 1 then this cache hit would not have occurred, and the block

containing the byte at address 1 would have been loaded from main memory.  The

tutorial now continues by illustrating the same effect of the increased block size,

this time with CPU-issued data read addresses of 2 and 3.

Screen 5

This is the end of tutorial 4.  As you have seen, increasing the block size also

increases the chance of a cache hit occurring (see explanation of the Principle of

Locality in the HASE Dinero user guide).  If the cache block size had been set to

4 in this tutorial, then only 1 cache miss would have occurred, for the first data

read of address 0.  The remaining data read requests for addresses 1, 2 and 3 would

all have been contained in the block loaded for address 0.  However, note that in a

real hardware cache implementation, the time taken to load a block from main

memory to the cache increases with block size, so there is a trade-off to be made.

Tutorial 5

Screen 1

Tutorial 5 demonstrates how HASE Dinero represents a cache that has been

split from a unified cache to separate data and instruction caches.  The cache

configuration has been set to direct-mapped with a block size of 1.  Both data

and instruction caches can store up to 4 bytes.  You will need to open both the

Data Cache Contents window and the Instruction Cache Contents window for

this tutorial.

Screen 2

To demonstrate the operation of a split cache in this tutorial, the CPU issues four

address references to the cache.  The addresses alternate between data reads and

instruction fetches.  Note that data read requests are always directed to the Data

Cache, whereas instruction fetch requests are always directed to the Instruction

Cache.

Screen 3

The CPU has just issued an instruction fetch command.  Therefore, the Cache

Control Unit will check only the Instruction Cache to see if the required data is

present.  To represent that the Cache Control Unit is accessing the Instruction

Cache in the Cache Memory Array, the ACTIVE_CACHE display on the Cache

Control Unit has been set to INSTRUCTION.

Screen 4

The block loaded from main memory has been written to line 0 in the Instruction

Cache.  The Cache Control Unit treats both the Data and Instruction caches as though

they are identical to a Unified Cache, with the only difference being which type of

CPU request each cache services.

Screen 5

This time the CPU has issued a data read request.  Therefore the ACTIVE_CACHE

display on the Cache Control Unit has been set to DATA.

Screen 6

Notice that the block loaded from main memory has been written to the Data Cache

this time.  The remainder of this tutorial consists of the CPU issuing one more

instruction fetch command and one more data read command, to give you a chance

to familiarise yourself with the way the different requests are handled.

Screen 7

This is the end of tutorial 5.  Hopefully you can now see how the cache handles

separate Data and Instruction storage areas, but you may be wondering what the

benefit of splitting the cache is.  The answer is that for a real hardware

implementation of a cache, each cache type (data and instruction) has a separate

connection to the CPU.  This allows the CPU to access both caches simultaneously

which obviously brings a performance improvement.  Although this concurrent access

capability is not shown in HASE Dinero, an illustration of how the Cache Control

Unit handles the cache split is shown.

Tutorial 6

Screen 1

Tutorial 6 is the first of three tutorials demonstrating the cache replacement

policies available in HASE Dinero, showing how the Least-Recently-Used

(LRU) replacement policy works.  This tutorial requires the Unified Cache

Contents screen to be open.

Screen 2

The cache modelled in this tutorial is fully associative and has a 1 byte block

size.  Before the replacement policy can be demonstrated, the cache needs to

be filled.  The following section of the tutorial shows the fully associative

cache being completely filled by CPU data read references.

Screen 3

A cache hit has just occurred to cache line 1.  Notice that this has changed the

timestamp field of the cache line so that it now has a higher value than cache line

2, even though the contents of cache line 1 were loaded to the cache before the

contents of cache line 2.  This is because the Cache Control Unit updates the

timestamp of each cache line every time it is accessed, not just when it is first

loaded.  The following CPU reference fills the last remaining empty cache line.

Screen 4

The cache is now full, so any additional blocks that are loaded to the cache in

response to CPU memory accesses will need to overwrite existing valid cache

lines when they are stored in the cache.  In this tutorial, the replacement policy

is set to LRU (Least Recently Used), which means that the cache line with the lowest

timestamp value will be chosen for replacement.  Note that as the cache is fully

associative, the timestamps of every cache line are checked when determining which

cache line to overwrite.  However, if the cache was set associative then only the lines

in one associative set would be eligible locations to store the new block.

Screen 5

The latest CPU reference, a data read for memory location 4 will require a LRU

replacement to be made.  The block loaded from main memory for the current cache

operation will be stored on cache line 0, overwriting the existing data, because this is

the line with the lowest timestamp value.  The next data read reference operation will

also require a LRU replacement to be made.

Screen 6

The block loaded from main memory for the current cache operation will overwrite

1 of the cache lines currently containing valid data.  Cache line 2 will be chosen, as

it has the lowest timestamp value.  Note that the LRU ordering has been affected by

the cache hit that occurred 3 program lines previously.  If the cache hit had not

occurred then the line with the lowest timestamp now would be line 1.

Screen 7

This is the end of tutorial 6.  You should now understand how the LRU replacement

policy operates, and in particular the effect that cache hits have on the ordering

of LRU replacements.  The fact that cache hits do affect the ordering of cache line

replacement is the only distinguishing factor between LRU and the First-In, First-Out

(FIFO) policy, which is demonstrated in the next tutorial.

Tutorial 7

Screen 1

Tutorial 7 demonstrates the operation of the First-In, First-Out (FIFO) cache

replacement policy.  The sequence of memory references issued by the CPU

in this tutorial is the same as for the previous tutorial so that a comparison can

be made between the LRU and FIFO replacement policies.  This tutorial requires

that the Unified Cache Contents window be open.  You should observe during the

course of the tutorial that FIFO ordering is maintained by always inserting new

entries in the cache to line 0, with any existing cache lines being moved down by

1 position, potentially resulting in cache lines being removed from the cache.

Screen 2

The primary aim of this tutorial is to demonstrate how the cache hit that occurs

on program line 4 has no effect on the ordering of FIFO replacements, unlike the

previous tutorial that used the LRU replacement policy.  The cache modelled in this

tutorial is fully associative and has a 1 byte block size.  Before the replacement policy

can be demonstrated, the cache needs to be filled.  The following section of the tutorial

shows the fully associative cache being completely filled by CPU data read references.

Screen 3

The cache is now full, so any additional blocks that are loaded to the cache in

response to CPU memory accesses will need to overwrite existing valid cache

lines when they are stored in the cache.  In this tutorial, the replacement policy

is set to FIFO (First-In, First-Out), which means that the oldest cache line (in terms of

when the cache line block was loaded to the cache) will be chosen for replacement.

Screen 4

The latest CPU reference, a data read for memory location 4 will require a FIFO

replacement to be made.  The block loaded from main memory for the current cache

operation will be stored in cache line 0, causing the block currently stored in cache

line 3 to be lost from the cache.  The next data read reference operation will also

require a FIFO replacement to be made.

Screen 5

The block loaded from main memory for the current cache operation will cause the

block currently stored in cache line 3 to be removed from the cache.  All other cache

lines will be moved down by 1 position to make room for the new cache line to be

stored in cache line 0.  Note that the LRU replacement policy would have caused the

block currently stored in line 2 to be removed from the cache as it has the lowest

timestamp of any of the cache lines.

Screen 6

This is the end of tutorial 7.  You should now understand how the FIFO replacement

policy operates, and in particular the difference between FIFO and LRU

replacement policies: whether replacements are made on the basis of ordering of

block-loads to the cache (FIFO) or on the basis of accesses to cache lines (LRU).

Tutorial 8

Screen 1

Tutorial 8 demonstrates the Random replacement policy.  The tutorial requires

that the Unified Cache Contents window be open.  Note that it will be useful

to have observed the prior 2 tutorials before this one, as the tutorial will

then be easier to understand.

Screen 2

The same trace file as the previous 2 tutorials is used here, so that a comparison

can be made between LRU, FIFO and random replacement.  However, the trace

file used in this tutorial has 3 extra CPU data read requests, so that the random

nature of line replacement is better demonstrated.  There is no further

explanatory text in this tutorial is the operation of random replacement is very

straightforward.

Screen 3

This is the end of tutorial 3.  You should now have an understanding of how

random replacement works and, if you have seen tutorials 6 and 7, also

understand how random replacement differs from LRU and FIFO.

Tutorial 9

Screen 1

Tutorial 9 demonstrates how the Copy-Back Write Policy operates.  You

will need to open the Unified Cache Contents window for this tutorial.

Note that if the Copy-Back policy is chosen then the Write Allocation

Policy must be set to Write, otherwise invalid data will be written to main

memory (see the HASE Dinero user guide if you are unclear on this point).

Screen 2

The cache used in this tutorial is 8-byte direct-mapped with a block size of 2.

A demonstration of 2 situations relevant to the copy-back write policy will be

provided.  Firstly, the actions taken on a CPU write, and secondly the actions

taken when a modified line in the cache is replaced.  Note that the Swap Data

field in the Status window is set to either YES or NO as the Cache Control Unit

determines whether a block needs to be written to main memory due to a modified

line being chosen for replacement.

Screen 3

The CPU has just issued a write instruction.  All write instructions cause a block to

be loaded from main memory concurrently with the Cache Control Unit reading the

Cache Memory Array.  This is done so that, in the event of a cache miss, the remaining

part of the block containing the byte written by the CPU is available to be written to the

Cache Memory Array.  Note that if a cache hit occurs for a CPU write then the block

loaded from main memory is not used.

Screen 4

The Cache Control Unit has just written the block containing the modified byte, from

the CPU, to line 0 of the cache.  Note that the first byte in the block has a negative

sequence number.  This indicates that the byte is modified and originates from the

CPU.  The second byte in the block has a positive sequence number to indicate that it

originates from main memory.  More importantly, the modified bit for cache line 0

has been set to 1, indicating that the cache line contents cannot be discarded without

writing the block to main memory.

Screen 5

The data read command just issued by the CPU contains an address of 8.  Due to the

cache configuration (direct-mapped, 8-bytes in size), the block loaded from main

memory to service the cache miss must be stored in line 0 of the cache, thus

replacing the modified block stored there.  As a consequence, the modified block will

be written to main memory in parallel with the Cache Control Unit writing the new

block to the Cache Memory Array and sending the requested byte to the CPU.

Screen 6

This is the end of tutorial 6.  You should now understand how the copy-back write

policy affects both CPU writes and the discarding of cache lines.  The following

2 tutorials demonstrate how the Write-Through, Write Allocate and Write-Through,

No-Write Allocate policies work.

Tutorial 10

Screen 1

Tutorial 10 demonstrates the operation of a Write Through, Write Allocate

cache write policy.  You will need to open the Unified Cache Contents

window for this tutorial.  This tutorial should be viewed in conjunction with

tutorial 11, which demonstrates how changing the allocation policy to No

Write affects the Write Through write policy.

Screen 2

The cache configuration in this tutorial is 8-byte direct-mapped with a 2

byte block size.  The tutorial demonstrates two features of the modelled

write policy.  Firstly, actions taken if the CPU write causes a cache hit are

shown.  Secondly, actions taken if the CPU write causes a cache miss are

shown.

Screen 3

The first program line is a data read and loads a block to the cache so

that a write hit can occur for the next program line.

Screen 4

The data write command just issued by the CPU has an address of 0 and

so will cause a hit to cache line 0.  As for the copy-back policy, this will

cause the data stored at line 0 to be modified.  However, the modified

block is also immediately written to main memory.  As a consequence, the

modified bit for cache line 0 is NOT set.

Screen 5

The data write command just issued by the CPU has an address of 2 and

so will cause a cache miss.  Because the Write-Through policy is set to

Allocate, the data written by the CPU will be written both to main memory

AND the cache.  If the Allocation policy had been set to No-Allocate, the

data written by the CPU would ONLY be written to main memory, not the

cache.

Screen 6

This is the end of tutorial 10.  You should now understand how the Write-

Allocation policy differs from Copy-Back, in terms of writing modified data

to main memory, and also how the Write Allocate policy affects Write-Through.

Tutorial 11

Screen 1

Tutorial 11 demonstrates the operation of a Write Through, No Write Allocate

cache write policy.  You will need to open the Unified Cache Contents

window for this tutorial.  This tutorial should be viewed after tutorial 10, so that

the effect of changing the allocation policy from Write to No Write can be observed.

Screen 2

The cache configuration in this tutorial is the same as for the previous tutorial

(8-byte direct-mapped with a 2 byte block size).  In addition, the same sequence

of CPU addresses as the previous tutorial is used.  The tutorial demonstrates two

points.  Firstly, the effect of a cache hit for a CPU write with Write-Through, No

Write allocate is the same as for Write-Through, Write.  However, the effect of

a CPU write miss for Write-Through No-Write is different to Write-Through, Write.

Screen 3

The data write command just issued by the CPU has an address of 0 and

so will cause a hit to cache line 0.  As for the Write-Through, Write policy, this

will cause the block stored at line 0 to be updated (as well as updating main memory).

Screen 4

The data write command just issued by the CPU has an address of 2 and

so will cause a cache miss.  Because the Write-Through policy is set to

No-Allocate, the data written by the CPU will be written ONLY to main memory.

If the Allocation policy had been set to Allocate, then the data written by the CPU

would also be written to the cache.

Screen 5

This is the end of tutorial 11.  You should now understand the difference between the

following cache write policies:

Copy-Back, Write Allocate

Write-Through, No-Write Allocate

Write-Through, Write Allocate.

Tutorial 12

Screen 1

Tutorial 12 demonstrates how prefetching activity is handled in the HASE Dinero

simulation.  This is one of the most complicated tutorials, and assumes that you are

familiar with the content of all of the previous tutorials.  The tutorial requires that

the Prefetch Buffer and Unified Cache Contents windows both be open.  The cache

configuration for the purpose of this tutorial is an 8 byte direct-mapped cache with

a 1 byte block size.  The write policy is set to Copy-Back.

Screen 2

The Fetch policy is set to ALWAYS PREFETCH for this tutorial.  This means

that for every READ (not write) request from the CPU, a prefetch operation will

occur.  The Prefetch stride has been set to 1.  This means that every prefetch operation

will fetch from main memory a block located 1 place sequentially higher in main

memory than the block required to service the current CPU read operation.  In addition,

the prefetch buffer is able to store 2 items in this demonstration.  Note that the prefetch

buffer uses a circular FIFO policy to replace items as new data is loaded.

Screen 3

The CPU has just issued a Fetch Instruction.  This will result in a prefetch command

being sent to main memory from the Cache Control Unit.  Main memory will respond

by sending the prefetch data to the Cache Control Unit.  Note the prefetch address /

data links have separate ports on the Main Memory and Cache Control Unit.  The

data sent by Main Memory to the Cache Control Unit will be stored in the Prefetch

Buffer.  See the HASE Dinero user guide to find out the format of Prefetch Buffer

lines.  Every time a cache miss occurs, the prefetch buffer will be checked for a hit by

the Cache Control Unit BEFORE the required block is loaded from Main Memory.

Screen 4

The CPU has just issued a Data Read command.  As for the previous Instruction Fetch

command, this will result in a prefetch command being issued by the Cache Control

Unit, with the resulting prefetch data being stored in the 2nd line of the Prefetch Buffer.

Screen 5

The CPU has just issued a Data Write command.  This has been included so that you

can observe that no prefetch operations are issued for data write commands.

Screen 6

The CPU has just issued a Data Read command.  The block required to service this

Data Read is stored in the Prefetch Buffer.  Consequently, when a cache miss occurs

for the Data Read, the required block will be loaded from the Prefetch Buffer, instead

of Main Memory.  Note also that the Data Read causes a prefetch operation to occur.

The data loaded from Main Memory by the prefetch command is stored in line 1 of the

Prefetch Buffer (due to the circular FIFO replacement policy), and overwrites the

data actually used to cause the Prefetch Buffer hit.  Note that this replacement occurs

AFTER the old data has been read from the Prefetch Buffer.

Screen 7

The CPU has just issued a Data Write command with an address of 2.  Note that the

block containing this address is already contained in the cache AND the Prefetch

Buffer.  However, the data stored in the Prefetch Buffer will be invalid after the

CPU write has taken effect and so needs to be invalidated.  You will therefore notice

the Valid Bit of line 0 in the Prefetch Buffer being set to 0 as the Data Write occurs.

Screen 8

This is the end of tutorial 12.  You should now understand how prefetching operates

and the benefit it brings to the cache hit ratio.  Performing prefetch operations can

double the hit ratio of a direct-mapped cache.  However, in a real hardware

implementation of the cache, the benefit of prefetching is offset against the extra

bandwidth required between main memory and the cache so that prefetch operations

can occur.

Tutorial 13

Screen 1

Tutorial 13 demonstrates how the MISS_PREFETCH Fetch Policy operates.

It is essential that you have already run and understood tutorial 12 before

attempting this tutorial.  You will need to have both the Prefetch Buffer and Unified

Cache Contents windows open.  The cache configuration is the same as for the previous

tutorial: 8-byte, direct-mapped, block size of 1 and Copy-Back writing.  As for the

previous tutorial, the Prefetch Buffer can store 2 items and the prefetch stride is set to

1 block.

Screen 2

This is the only descriptive text for this tutorial as the difference between Always

Prefetch and Miss Prefetch is very simple.  In contrast to the Always Prefetch

Policy, Miss Prefetch does not generate a prefetch command if  a cache hit occurs.

Note that if a cache miss occurs, but a prefetch hit does occur then a prefetch operation

will still be issued – the prefetch operation is only cancelled if a hit occurs to the main

cache memory.

Screen 3

This is the end of tutorial 13.  Hopefully, you now understand the difference between

Always Prefetch and Miss Prefetch.

Tutorial 14

Screen 1

Tutorial 14 demonstrates the benefit to be gained from adding a victim cache to

a cache design.  This tutorial assumes that you are familiar with all previous

tutorials.  You will need to have the Unified Cache Contents and Victim Cache

windows open.  The cache configuration used in this demonstration is as follows:

8 bytes, direct-mapped, 1 byte block.

Screen 2

A Victim Cache stores all blocks that are discarded from the cache, overwriting

existing lines in the Victim Cache using a circular FIFO replacement policy.  For

the purposes of this tutorial, the victim cache can only store 1 block.  You will

observe a block being discarded from the cache, due to the direct-mapped

replacement policy, and stored in the Victim Cache.  The block stored in the

Victim Cache will then be used to service a CPU Read command.  The current

Data Read command loads a block to line 0 of the cache.

Screen 3

The current Data Read command from the CPU has an address of 8.  Due to the

direct-mapped associativity of the cache, address 8 maps to cache line 0 and so

the existing block in line 0 will be removed from the cache, and stored in the

Victim Cache.

Screen 4

The current Data Read command from the CPU has an address of 0.  This will

cause a cache miss, but will cause a hit to the Victim Cache, thus demonstrating

the benefit of the Victim Cache to cache designs with a low associativity.  Notice

that when the Victim Cache hit occurs, the block is loaded from the Victim Cache

to the main cache array, causing the existing block in line 0 to be replaced.  This

replaced block will, in turn, be stored in the Victim Cache.  Thus a swap operation

occurs with the hit location in the Victim Cache and the cache line chosen for

replacement (in this example, there is only 1 choice as the cache is direct-mapped).

Screen 5

This is the end of Tutorial 14, and concludes the Tutorial Section of HASE Dinero.

Whilst all of the cache features have been demonstrated during the course of the

tutorials, they have been shown in isolation.  It is of course possible to use all

of features in combination, although the animation of the display will become

complicated.  Note also that the cache sizes chosen for the tutorials (8 bytes) are

far smaller than any realistic cache, and were chosen to be so small for the benefit

of clarity.  If you would like to experiment with cache design yourself, simply set

the Simulation Mode to NORMAL.